- Human Command Interface

The receive routine masks bit 7 of all characters received and buffers and echoes all printable ASCII characters. <TAB>s are converted to <SPACE>s. Several other control characters are recognized or transmitted:

| Name | Designation | HEX | Operation |
| --- | --- | --- | --- |
| Backspace | <BKSP> | 0816 | Queue pointer decremented; echoed |
| TAB | <TAB> | 0916 | Converted to <SPACE> |
| Line Feed | <LF> | 0A16 | Placed in queue, not echoed, otherwise ignored |
| Carriage Return | <CR> | 0D16 | Placed in queue, not echoed, command terminator |
| DC1 – XON | <CTRL-Q> | 1116 | Flow Control – Re-enable xmtr/rcvr |
| DC3 – XOFF | <CTRL-S> | 1316 | Flow Control – Disable xmtr/rcvr |
| Escape | <ESC> | 1B16 | Not placed in queue. Terminates current operations, flush queue |

A number of control characters are set up as hot keys to perform the function of the Mode, Pattern, etc. buttons on the PCB and on the rear panel switch via the serial interface. No <CR> terminator is required for these commands to execute. See the first entry in Section 3.2 of the commands descriptions.

2.1	The ! Hot Key

Typing or sending a single exclamation point (!) immediately after the prompt with no whitespace or control characters – will repeat the last command (the exclamation point is not echoed). If ANY characters are sent between the prompt and the “!” – even a <CR> - the previous command will be lost and cannot be repeated using the “!” option. “!” just re-processes the current contents of the command buffer so it will also repeat illegal commands or commands with illegal or missing

parameters.

  - ASCII Control Characters

In almost all cases sending an <ESC> character will terminate current operations and return a command prompt. It can also be used when sending commands manually through a terminal to cancel any characters typed in after the prompt.

Whitespace characters - <TAB> and <SPACE> - are treated identically. Line feeds - <LF> - are queued but are otherwise

ignored by the code. A “newline” is always a <CR><LF> pair. The prompt is a newline followed by a > and a space.

Single <BKSP> characters are echoed and back up the queue pointer. A <BKSP> sent to the projector is not converted to a “destructive” backspace string - <BKSP><SPACE><BKSP> when echoed. Only a single ASCII <BKSP> will be echoed.

If the terminal program can be configured to send <BKSP><SPACE><BKSP> when the Backspace Key is pressed, this

option should be selected.

Commands sent to the board in either mode can be terminated with either a single <CR> or a <CR><LF> pair. The <LF>

character triggers command parsing and execution. The <LF> is ignored. Neither commands nor hex data is case-sensitive.

  - ZOOM, FOCUS and Lens Shift Commands

| ZOOM, FOCUS and Lens Shift Serial Commands |  |
| --- | --- |
| The following commands turn on the specified motors. The motors will stay energized until an <ESC> character is sent to the projector. The last character of each command is an ASCII digit from 0 to 3 with no intervening whitespace. This determines motor speed with 0 being the slowest and 3 the fastest. All of the motors have clutches or limit switches to prevent damage if they are allowed to run after reaching a mechanical limit. |  |
| Command 0 ≤ n ≤ 3 | Description |
| Dn | Move image DOWN |
| Ln | Move image to the LEFT |
| FFn | Move FOCUS away from Projector – FOCUS FAR |
| FNn | Move FOCUS toward Projector – FOCUS NEAR |
| Rn | Move image to the RIGHT |
| Un | Move image UP |
| ZIn | ZOOM IN – Projected Image Smaller |
| ZOn | ZOOM OUT – Projected Image Larger |
| The following NUDGE commands turn on the specified motors for a short period. No <ESC> character need be sent. The last character of each command is an ASCII digit from 0 to 3 with no intervening whitespace. This determines period the motor is energized with 0 being the shortest and 3 the longest. |  |
| Command 0 ≤ n ≤ 3 | Description |
| NDn | Nudge image DOWN |
| NLn | Nudge image to the LEFT |
| NFFn | Nudge FOCUS away form Projector – FOCUS FAR |
| NFNn | Nudge FOCUS toward Projector – FOCUS NEAR |
| NRn | Nudge image to the RIGHT |
| Nun | Nudge image UP |
| NZIn | Nudge ZOOM IN – Nudge Projected Image Smaller |
| NZOn | Nudge ZOOM OUT – Nudge Projected Image Larger |

  - User Commands

| User Serial Commands |  |
| --- | --- |
| Command | Brief Description |
| CLOSE | Close Shutter and put Formatters in Curtain Mode |
| DGAMMA | DGAMMA Dowlnoad |
| FPOL | TOGGLE the Film Projector Lockout Polarity |
| Gamma | Select Gamma Table |
| GETP | Get IR Remote |
| IR+ | Enable IR Remote |
| IR- | Disable IR Remote except POWER Button (default at Power-up) |
| LH | Display Lamp 1 or 2 Age Data |
| LAMPON1 | Turn On Lamp1* |
| LAMPON2 | Turn On Lamp2* |
| LAMPOFF1 | Turn Off Lamp1* |
| LAMPOFF2 | Turn Off Lamp2* |
| M+ | If V- enable display of non-critical (informational) messages – preceded by % |
| M- | If V- disable display of non-critical (informational) messages – preceded by % |
| OFF | Main Power Shutdown |
| ON | Main Power ON |
| ONS | Main Power ON and Start Signal Lamp mode |
| OND | Main Power ON and Start Dual Lamp mode* |
| OPEN | Open Shutter and put Formatters in Normal Projection Mode |
| P ID | Check Projector Serial Number |
| SIZE | Set Display Size |
| SETP | Set Lamp Power Mode* |
| STS | Check Shutter Status |
| V+ | Enable Verbose Mode |
| V- | Disable Verbose Mode (see M+ and M- above also) |
| VER | Display Controller Firmware Revision |

  - Enhanced Color Correction Commands

| DEBUG/Maintenance Serial Commands |  |
| --- | --- |
| Command | Brief Description |
| ECC | Enable/Disable P7 Color Correction and Select Target Data |
| MDNLD | Download Measured Color Gamut Data for Storage in BLUE Formatter EEPROM |
| RDMCGD | Display Measured Color Gamut Data |
| RDTCGD | Display Specified Target Color Gamut Data |
| TDNLD | Downlaod User-Defined Target Color Gamut Data for Storage in EEPROM |

  - DEBUG/Maintenance Commands

| DEBUG/Maintenance Serial Commands |  |
| --- | --- |
| Command | Brief Description |
| BAUD | Change immediate and power-up baud clock |
| FH | Display the FORMATTER Register Help Table |
| FR | Read Register from ALL Formatters |
| FRB | Read BLUE Formatter Register |
| FRG | Read GREEN Formatter Register |
| FRR | Read RED Formatter Register |
| FW | Write Register to ALL Formatters |
| FWB | Write to BLUE Formatter Register |
| FWG | Write to GREEN Formatter Register |
| FWR | Write to RED Formatter Register |
| OCMD | Send Command to Ostram Ballast and Display Response |
| RST | Reset and Reinitialize Formatters |

1 : mark for MH-69 only.